The Reel Experts

INSTRUCTIONS

Congratulations, you’ve just purchased the ultimate dive reel!!  We would like at this time to discuss a few of the features to make you more familiar with this product.

The MantaReel™ is constructed of anodized aluminum, stainless steel and UHMW plastic.  While these materials are all highly corrosive resistant, we recommend to prolong the life of your reel, rinse it with fresh water after every dive. Caution: Winding line tight when wet causes line to stretch. As it dries the line shrinks which may cause spool to spread or become difficult to turn. Also place a drop of oil or WD40 on the trigger slot (Jr. & Sr.) reels or brake screw (Mini & SD) reels.

The Wave Washer Tension Control:  The purpose of the wave washer is to keep the line on your reel from free-spooling or “birdsnesting”.  The wave washer tension control is set at the factory with a medium amount of tension.  If you would like to alter the amount of tension follow these instructions carefully.  Tools needed are a 9/16-inch box wrench and 9/16 inch socket wrench (Field Adjustment) A bench vise and 9/16 socket wrench are preferred.  NOTE: Tension control does not usually require adjustment and should not be adjusted until after you have used the reel. Water acts as a lubricant on tension control.

- To adjust the amount of tension, place head of spool bolt in bench vise or hold securely with box wrench. With bolt head secure loosen nylon lock nut. When nut is loose rotate frame in small increments (1/6 turn) clockwise to increase tension, counter-clockwise to decrease. You may test tension by holding frame stationary and rotating spool

- After making adjustment, retighten nylon lock nut securely while holding bolt head stationary.  .   . Note:  It is extremely important when tightening nylon lock nut that the bolt head does not   move.

With adjustment complete wind your reel for 30 - 40 cranks to make sure it functions properly. If tension does not hold repeat steps 1 & 2.

CAUTION:

BEFORE USING THIS EQUIPMENT YOU SHOULD BE PROPERLY TRAINED.  THIS PRODUCT IS FOR ADVANCED DIVERS, TO OBTAIN THE PROPER TRAINING NEEDED, CONTACT YOUR LOCAL DIVE SHOP.

Warranty

Manta's Limited Warranty: Five years from date

of original purchase, Manta Ind. will cover

the following without charge with mailed

registration of purchase. Any replacement and

adjustments of defective parts needed for said

product to conform fully to factory prescribed

standards.